SpeXtra: 426338

Hardware Group No. 1.0

SGL 2' 4" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 001/N0-001a | 035/N0-048 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 3 | EA | HINGE | 5BB1 4.5 X 4.5 | 652 | IVE |
| 1 | EA | CLASSROOM LOCK | ND70PD RHO XN12-307 | 626 | SCH |
| 1 | EA | SURFACE CLOSER | 1461 RW/PA | 689 | LCN |
| 1 | EA | FLOOR STOP | FS13 | US26D | IVE |
| 1 | EA | GASKETING | 8303AA-S 30 X 84" (762 X 2134MM) | AA | ZER |
| 1 | EA | DOOR SWEEP | 111AA 30" (762MM) | AA | ZER |
| 1 | EA | THRESHOLD | 544A-223-30" (762MM) | A | ZER |

Operational Description

5 Knuckle Ball Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Outside lever locked and unlocked by key. Inside lever always unlocked. Inside lever is always free for immediate egress.
Surface Mount, Non Handed, Universal, Mechanical Closer , Mount as Required, Regular w/ Parallel Arm Brkt, Slim Line Cover, Self Reaming & Tapping Screws
Dome Floor Stop

Hardware Group No. 10.0

SGL 2' 0" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 087/N0-042 | 088/N0-043 | 089/N2-034 | 090/N2-035 | 091/N0-049A |  |
| --- | --- | --- | --- | --- | --- |

SGL 2' 6" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 071/N0-003A | 072/N0-015 | 073/N1-016 | 074/N0-023 | 075/N0-024 | 076/N1-017 |
| --- | --- | --- | --- | --- | --- |
| 077/N1-021 | 078/N1-026 | 079/N1-027 | 080/N1-037 | 081/N2-007 | 082/N2-010 |
| 083/N2-011 | 084/N2-014 | 085/N2-019 | 086/N2-031 |  |  |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 3 | EA | HINGE | 5PB1 4.5 X 4.5 | 652 | IVE |
| 1 | EA | PRIVACY LOCK | ND40S RHO | 626 | SCH |
| 1 | EA | FLOOR STOP | FS13 | US26D | IVE |
| 3 | EA | SILENCER | SR64 | GRY | IVE |

Operational Description

5 Knuckle Plain Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Push-button locking. Can be opened from outside with small screwdriver. Turning inside lever or closing door releases button. Inside lever is always free for immediate egress.
Dome Floor Stop
Door Silencer

Hardware Group No. 11.0

SGL 3' 0" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 092/N0-013 | 093/N0-036 | 094/N0-038 | 095/N0-039 | 096/N1-024 | 097/N1-025 |
| --- | --- | --- | --- | --- | --- |
| 098/N1-036 |  |  |  |  |  |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 3 | EA | HINGE | 5BB1 4.5 X 4.5 | 652 | IVE |
| 1 | EA | DBL CYL DEADBOLT | B562P | 626 | SCH |
| 1 | EA | PUSH PLATE | 8200 4" X 16" | 630 | IVE |
| 1 | EA | PULL PLATE | 8305 10" 4" X 16" | 630 | IVE |
| 1 | EA | SURFACE CLOSER | 1461 RW/PA | 689 | LCN |
| 1 | EA | KICK PLATE | 8400 12" X 34" B-CS | 630 | IVE |
| 3 | EA | SILENCER | SR64 | GRY | IVE |

Operational Description

5 Knuckle Ball Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Dbl Cyl Deadbolt
Push Plate, 4X16"FALSE
Door Pull Complete with Back Plate, 10, 4X16" Mounting Type STDFALSE
Surface Mount, Non Handed, Universal, Mechanical Closer , Mount as Required, Regular w/ Parallel Arm Brkt, Slim Line Cover, Self Reaming & Tapping Screws
Door Silencer

Hardware Group No. 12.0

PR 8' 0" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 099/N0-012 | 100/N0-030 | 102/N1-004 | 109/N1-018 | 112/N1-023 | 117/N1-038 |
| --- | --- | --- | --- | --- | --- |
| 119/N2-012 | 120/N2-013 | 121/N2-021 | 123/N2-027 | 124/N2-041 |  |

Provide each PR door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 6 | EA | HINGE | 5BB1 4.5 X 4.5 | 652 | IVE |
| 2 | EA | MANUAL FLUSH BOLT | FB358 | 626 | IVE |
| 1 | EA | DBL CYL DEADBOLT | B562P | 626 | SCH |
| 2 | EA | PUSH/PULL BAR | 9190EZHD-12"-NO | 630-316 | IVE |
| 2 | EA | SURFACE CLOSER | 1461 RW/PA | 689 | LCN |
| 2 | EA | KICK PLATE | 8400 34" X 34" B-CS | 630 | IVE |
| 1 | EA | GASKETING | 188SBK 20FT PSA | BK | ZER |
| 2 | EA | DOOR SWEEP | 8194AA 36" (914MM) | AA | ZER |

Operational Description

5 Knuckle Ball Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Manual Flush Bolt for Wood Door
Dbl Cyl Deadbolt
90 degree offset door pull, 2-1/2" clearance and horizontal push bar combination
Surface Mount, Non Handed, Universal, Mechanical Closer , Mount as Required, Regular w/ Parallel Arm Brkt, Slim Line Cover, Self Reaming & Tapping Screws

Hardware Group No. 13.0

SGL 4' 0" X 7' 0" X 1 3/4" X UNK X HMF X NONRTD

For use on mark/door #(s):

| 101/N1-003 | 103/N1-005 | 114/N1-029 | 115/N1-032 | 126/N1-046 |  |
| --- | --- | --- | --- | --- | --- |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 1 | EA | DBL CYL DEADBOLT | B562P | 626 | SCH |
| 1 | EA | PUSH/PULL BAR | 9190EZHD-12"-NO | 630-316 | IVE |
| 1 | EA | SURFACE CLOSER | 1461 H | 689 | LCN |
| 1 | EA | KICK PLATE | 8400 12" X 46" B-CS | 630 | IVE |
| 1 | EA | GASKETING | 188SWH 22FT PSA | WH | ZER |
| 1 | EA | DOOR SWEEP | 8194AA 48" (1219MM) | AA | ZER |

Operational Description

Dbl Cyl Deadbolt
90 degree offset door pull, 2-1/2" clearance and horizontal push bar combination
Self-Closing

Hardware Group No. 14.0

SGL 3' 0" X 7' 0" X 1 3/4" X HMD X HMF X NONRTD

For use on mark/door #(s):

| 106/N1-012 | 107/N1-013 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 3 | EA | HINGE | 5PB1 4.5 X 4.5 | 652 | IVE |
| 1 | EA | PUSH PLATE | 8200 4" X 16" | 630 | IVE |
| 1 | EA | PULL PLATE | 8305 10" 4" X 16" | 630 | IVE |
| 1 | EA | SURFACE CLOSER | 1461 RW/PA | 689 | LCN |
| 1 | EA | KICK PLATE | 8400 12" X 34" B-CS | 630 | IVE |
| 1 | EA | GASKETING | 8303AA-S 36 X 84" (914 X 2134MM) | AA | ZER |
| 1 | EA | DOOR SWEEP | 8194AA 36" (914MM) | AA | ZER |

Operational Description

5 Knuckle Plain Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Door Pull Complete with Back Plate, 10, 4X16" Mounting Type STDFALSE
Push Plate, 4X16"FALSE
Surface Mount, Non Handed, Universal, Mechanical Closer , Mount as Required, Regular w/ Parallel Arm Brkt, Slim Line Cover, Self Reaming & Tapping Screws

Hardware Group No. 15.0

PR 7' 0" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 104/N1-010 | 127/N1-006 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Provide each PR door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 2 | EA | PUSH PLATE | 8200 4" X 16" | 630 | IVE |
| 2 | EA | CONCEALED CLOSER | OHC100A S90 EL/DP | 695 | FAL |
| 4 | EA | KICK PLATE | 8400 34" X 34" B-CS | 630 | IVE |
| 2 | EA | DOOR SWEEP | 8194AA 36" (914MM) | AA | ZER |

Operational Description

Push Plate, 4X16"FALSE
Self-Closing

Hardware Group No. 16.0

PR 7' 0" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 105/N1-011 | 108/N1-014 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Provide each PR door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 1 | EA | DBL CYL DEADBOLT | B562P | 626 | SCH |
| 4 | EA | PUSH PLATE | 8200 4" X 16" | 630 | IVE |
| 2 | EA | CONCEALED CLOSER | OHC100A S90 EL/DP | 695 | FAL |
| 4 | EA | KICK PLATE | 8400 34" X 34" B-CS | 630 | IVE |
| 4 | EA | FLOOR STOP | FS13 | US26D | IVE |

Operational Description

Dbl Cyl Deadbolt
Push Plate, 4X16"FALSE
Self-Closing
Dome Floor Stop

Hardware Group No. 17.0

SGL 4' 0" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 110/N1-019 | 111/N1-020 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 1 | EA | DBL CYL DEADBOLT | B562P | 626 | SCH |
| 2 | EA | PUSH PLATE | 8200 4" X 16" | 630 | IVE |
| 1 | EA | CONCEALED CLOSER | OHC100A S90 EL/DP | 695 | FAL |
| 2 | EA | KICK PLATE | 8400 34" X 46" B-CS | 630 | IVE |

Operational Description

Dbl Cyl Deadbolt
Self-Closing

Hardware Group No. 18.0

SGL 3' 0" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 116/N1-033 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 3 | EA | HINGE | 5BB1 4.5 X 4.5 | 652 | IVE |
| 1 | EA | CLASSROOM LOCK | ND70PD RHO | 626 | SCH |
| 1 | EA | SURFACE CLOSER | 1461 H | 689 | LCN |
| 1 | EA | KICK PLATE | 8400 12" X 34" B-CS | 630 | IVE |
| 1 | EA | FLOOR STOP | FS13 | US26D | IVE |
| 3 | EA | SILENCER | SR64 | GRY | IVE |

Operational Description

5 Knuckle Ball Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Classroom Lock Cylindrical
Self-Closing
Dome Floor Stop
Door Silencer

Hardware Group No. 19.0

DE 7' 0" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 118/N2-004 | 122/N2-023 | 128/N2-033 |  |  |  |
| --- | --- | --- | --- | --- | --- |

Provide each DE door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 6 | EA | HINGE | 5BB1 4.5 X 4.5 | 652 | IVE |
| 2 | EA | PUSH PLATE | 8200 4" X 16" | 630 | IVE |
| 2 | EA | PULL PLATE | 8305 10" 4" X 16" | 630 | IVE |
| 2 | EA | SURFACE CLOSER | 1461 RW/PA | 689 | LCN |
| 2 | EA | KICK PLATE | 8400 34" X 34" B-CS | 630 | IVE |
| 2 | EA | SILENCER | SR64 | GRY | IVE |

Operational Description

5 Knuckle Ball Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Door Pull Complete with Back Plate, 10, 4X16" Mounting Type STDFALSE
Push Plate, 4X16"FALSE
Surface Mount, Non Handed, Universal, Mechanical Closer , Mount as Required, Regular w/ Parallel Arm Brkt, Slim Line Cover, Self Reaming & Tapping Screws
Door Silencer

Hardware Group No. 2.0

SGL 2' 4" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 002/N0-001B | 006/N1-045 | 007/N1-039 | 008/N1-040 | 011/N1-034 |  |
| --- | --- | --- | --- | --- | --- |

SGL 2' 8" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 009/N1-015 | 010/N1-016 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

SGL 3' 0" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 004/N0-006 | 005/N1-002 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 3 | EA | HINGE | 5BB1 4.5 X 4.5 | 652 | IVE |
| 1 | EA | CLASSROOM LOCK | ND70PD RHO | 626 | SCH |
| 1 | EA | SURFACE CLOSER | 1461 RW/PA | 689 | LCN |
| 1 | EA | FLOOR STOP | FS13 | US26D | IVE |
| 3 | EA | SILENCER | SR64 | GRY | IVE |

Operational Description

5 Knuckle Ball Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Classroom Lock Cylindrical
Surface Mount, Non Handed, Universal, Mechanical Closer , Mount as Required, Regular w/ Parallel Arm Brkt, Slim Line Cover, Self Reaming & Tapping Screws
Dome Floor Stop
Door Silencer

Hardware Group No. 20.0

SGL 3' 0" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 125/N0-041 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 3 | EA | HINGE | 5BB1 4.5 X 4.5 | 652 | IVE |
| 1 | EA | DBL CYL DEADBOLT | B562P | 626 | SCH |
| 1 | EA | PUSH PLATE | 8200 4" X 16" | 630 | IVE |
| 1 | EA | PULL PLATE | 8305 10" 4" X 16" | 630 | IVE |
| 1 | EA | SURFACE CLOSER | 1461 RW/PA | 689 | LCN |
| 3 | EA | SILENCER | SR64 | GRY | IVE |

Operational Description

5 Knuckle Ball Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Dbl Cyl Deadbolt
Door Pull Complete with Back Plate, 10, 4X16" Mounting Type STDFALSE
Push Plate, 4X16"FALSE
Surface Mount, Non Handed, Universal, Mechanical Closer , Mount as Required, Regular w/ Parallel Arm Brkt, Slim Line Cover, Self Reaming & Tapping Screws
Door Silencer

Hardware Group No. 3.0

SGL 3' 0" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 003/N0-002 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 3 | EA | HINGE | 5BB1 4.5 X 4.5 | 652 | IVE |
| 1 | EA | CLASSROOM LOCK | ND70PD RHO XN12-307 | 626 | SCH |
| 1 | EA | SURFACE CLOSER | 1461 RW/PA | 689 | LCN |
| 1 | EA | FLOOR STOP | FS13 | US26D | IVE |
| 1 | EA | GASKETING | 8303AA-S 36 X 84" (914 X 2134MM) | AA | ZER |
| 1 | EA | DOOR SWEEP | 111AA 36" (914MM) | AA | ZER |
| 1 | EA | THRESHOLD | 544A-223-36" (914MM) | A | ZER |

Operational Description

5 Knuckle Ball Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Outside lever locked and unlocked by key. Inside lever always unlocked. Inside lever is always free for immediate egress.
Surface Mount, Non Handed, Universal, Mechanical Closer , Mount as Required, Regular w/ Parallel Arm Brkt, Slim Line Cover, Self Reaming & Tapping Screws
Dome Floor Stop

Hardware Group No. 4.0

SGL 2' 8" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 026/N0-005 | 027/N0-007 | 028/N0-008 | 029/N0-009 | 030/N0-010 | 031/N0-035 |
| --- | --- | --- | --- | --- | --- |
| 032/N1-035 | 033/N2-005 | 034/N2-006 |  | 036/N0-049 |  |

SGL 3' 0" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 012/N0-003 | 013/N0-004 | 014/N0-011 | 015/N0-017 | 016/N0-018 | 017/N0-019 |
| --- | --- | --- | --- | --- | --- |
| 018/N0-020 | 019/N0-025 | 020/N0-026 | 021/N0-027 | 022/N0-028 | 023/N0-029 |
| 024/N0-033 | 025/N0-034 |  |  |  |  |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 3 | EA | HINGE | 5PB1 4.5 X 4.5 | 652 | IVE |
| 1 | EA | ENTRANCE LOCK | ND53PD RHO | 626 | SCH |
| 1 | EA | FLOOR STOP | FS13 | US26D | IVE |
| 3 | EA | SILENCER | SR64 | GRY | IVE |

Operational Description

5 Knuckle Plain Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Turn/push-button locking; pushing and turning button locks outside lever, requiring use of key until button is manually unlocked. Pushbutton locking; pushing button locks outside lever until unlocked by key or by turning inside lever. Inside lever is alwa
Dome Floor Stop
Door Silencer

Hardware Group No. 5.0

SGL 2' 6" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 037/N0-021 | 038/N1-001 | 039/N1-007 | 040/N2-009 | 041/N2-017 | 042/N2-025 |
| --- | --- | --- | --- | --- | --- |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 3 | EA | HINGE | 5PB1 4.5 X 4.5 | 652 | IVE |
| 1 | EA | STOREROOM LOCK | ND80PD RHO | 626 | SCH |
| 2 | EA | KICK PLATE | 8400 12" X 28" B-CS | 630 | IVE |
| 1 | EA | FLOOR STOP | FS13 | US26D | IVE |
| 3 | EA | SILENCER | SR64 | GRY | IVE |

Operational Description

5 Knuckle Plain Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Outside lever fixed. Entrance by key only. Inside lever always unlocked. Inside lever is always free for immediate egress.
Dome Floor Stop
Door Silencer

Hardware Group No. 6.0

SGL 2' 8" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 043/N0-022 | 044/N1-008 | 045/N1-030 | 046/N2-001 | 047/N2-002 | 048/N2-003 |
| --- | --- | --- | --- | --- | --- |
| 049/N2-022 | 050/N2-024 | 051/N2-029 | 052/N2-030 | 053/N1-042 | 054/N0-047 |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 3 | EA | HINGE | 5BB1 4.5 X 4.5 | 652 | IVE |
| 1 | EA | STOREROOM LOCK | ND80PD RHO | 626 | SCH |
| 1 | EA | SURFACE CLOSER | 1461 H | 689 | LCN |
| 1 | EA | KICK PLATE | 8400 12" X 30" B-CS | 630 | IVE |
| 1 | EA | FLOOR STOP | FS13 | US26D | IVE |
| 3 | EA | SILENCER | SR64 | GRY | IVE |

Operational Description

5 Knuckle Ball Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Outside lever fixed. Entrance by key only. Inside lever always unlocked. Inside lever is always free for immediate egress.
Self-Closing
Dome Floor Stop
Door Silencer

Hardware Group No. 7.0

SGL 3' 0" X 7' 0" X 1 3/4" X LPWD X HMF X NONRTD

For use on mark/door #(s):

| 055/N0-032 | 056/N0-037 | 057/N1-009 | 058/N1-028 | 059/N1-031 | 060/N2-016 |
| --- | --- | --- | --- | --- | --- |
| 061/N2-026 | 062/N0-044 | 065/N1-044 |  |  |  |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 3 | EA | HINGE | 5PB1 4.5 X 4.5 | 652 | IVE |
| 1 | EA | STOREROOM LOCK | ND80PD RHO | 626 | SCH |
| 1 | EA | KICK PLATE | 8400 12" X 34" B-CS | 630 | IVE |
| 1 | EA | FLOOR STOP | FS13 | US26D | IVE |
| 3 | EA | SILENCER | SR64 | GRY | IVE |

Operational Description

5 Knuckle Plain Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Outside lever fixed. Entrance by key only. Inside lever always unlocked. Inside lever is always free for immediate egress.
Dome Floor Stop
Door Silencer

Hardware Group No. 8.0

SGL 3' 0" X 7' 0" X 1 3/4" X LPWD X HMF X 90MIN

For use on mark/door #(s):

| 063/N0-045 | 064/N1-043 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 3 | EA | HINGE | 5BB1 4.5 X 4.5 | 652 | IVE |
| 1 | EA | STOREROOM LOCK | ND80PD RHO | 626 | SCH |
| 1 | EA | SURFACE CLOSER | 1461 RW/PA | 689 | LCN |
| 1 | EA | KICK PLATE | 8400 12" X 34" B-CS | 630 | IVE |
| 1 | EA | GASKETING | 8303AA-S 36 X 84" (914 X 2134MM) | AA | ZER |
| 1 | EA | DOOR SWEEP | 111AA 36" (914MM) | AA | ZER |
| 1 | EA | THRESHOLD | 544A-223-36" (914MM) | A | ZER |

Operational Description

5 Knuckle Ball Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Outside lever fixed. Entrance by key only. Inside lever always unlocked. Inside lever is always free for immediate egress.
Surface Mount, Non Handed, Universal, Mechanical Closer , Mount as Required, Regular w/ Parallel Arm Brkt, Slim Line Cover, Self Reaming & Tapping Screws

Hardware Group No. 9.0

SGL 3' 0" X 7' 0" X 1 3/4" X HMD X HMF X 90MIN

For use on mark/door #(s):

| 066/EXT-02 | 067/EXT-06 | 068/EXT-07 | 069/EXT-08 | 070/EXT-09 |  |
| --- | --- | --- | --- | --- | --- |

Provide each SGL door(s) with the following:

| Qty |  | Description | Catalog Number | Finish | Mfr |
| --- | --- | --- | --- | --- | --- |
| 3 | EA | HINGE | 5BB1 4.5 X 4.5 | 652 | IVE |
| 1 | EA | FIRE EXIT HARDWARE | 22-L-F-06-CYL | 689 | VON |
| 1 | EA | SURFACE CLOSER | 1461 RW/PA | 689 | LCN |
| 1 | EA | KICK PLATE | 8400 12" X 34" B-CS | 630 | IVE |
| 1 | EA | GASKETING | 8303AA-S 36 X 84" (914 X 2134MM) | AA | ZER |
| 1 | EA | DOOR SWEEP | 111AA 36" (914MM) | AA | ZER |
| 1 | EA | THRESHOLD | 544A-223-36" (914MM) | A | ZER |

Operational Description

5 Knuckle Ball Bearing Full Mortise Hinge Standard Weight 4.5"x4.5" 652
Free Egress at all times. Pressing Push Bar retracts latchbolts
Surface Mount, Non Handed, Universal, Mechanical Closer , Mount as Required, Regular w/ Parallel Arm Brkt, Slim Line Cover, Self Reaming & Tapping Screws